Techno-Economic and Prospective Life Cycle Assessment of Sustainable Propanol Production Pathways

Abhinandan Nabera,a Juan Diego Medrano-García,a Sachin Jog,a Robert Istrate,b and Gonzalo Guillén-Gosálbez*,a

aInstitute for Chemical and Bioengineering, Department of Chemistry and Applied Biosciences, ETH Zurich, Vladimir-Prelog-Weg 1, 8093 Zurich, Switzerland

bInstitute of Environmental Sciences (CML), Leiden University, Einsteinweg 2, 2333 CC Leiden, The Netherlands

*gonzalo.guillen.gosalbez@chem.ethz.ch

Abstract

The chemical industry has the highest energy demand across industrial sectors, primarily due to its heavy reliance on fossil fuels for both feedstock and utilities. Specifically, the industry consumes ca. 14% and 8% of the global oil and gas supply, respectively, contributing to 5.6 Gt CO2e emissions annually (including both direct and indirect emissions), which accounts for 10% of global greenhouse gas emissions (Bauer et al., 2023). To meet the ambitious targets set by the Paris Climate Agreement, numerous studies in recent years have focused on reducing CO2 emissions from chemical production. Techno-economic and environmental studies, in particular, have gained wide attention in identifying more sustainable production pathways, with life cycle assessment emerging as the prevalent tool for environmental impact assessments. However, most of the studies consider fixed background data, neglecting the effects that the future evolution of socio-economic systems could have on the chemical sector.

Propanol is a platform chemical with an annual demand of 4 Mt and a growth rate of 5%. Currently, propanol production relies on syngas and ethylene derived from fossil fuels, specifically from natural gas and naphtha, respectively (Vo et al., 2021). Consequently, the fossil-based production route for propanol results in significant environmental burdens. An alternative to the fossil-based route relies on using syngas from captured CO2 and renewable-powered electrolytic hydrogen via the reverse water-gas shift reaction. Alternatively, renewable carbon feedstocks such as biomass, biomethane, and plastics could also be used to produce syngas for propanol production. To date, the economic and environmental benefits of these alternative propanol production routes remain unexplored.

To fill this critical research gap, we analyse sustainable routes for producing propanol by conducting a techno-economic and prospective life cycle assessment to evaluate both their current and future environmental impacts. To this end, we develop detailed process simulations using Aspen HYSYS® v12.1 to quantify the process’s economic and environmental performance. For the environmental assessment, foreground data are extracted from the process simulation, while background inventories are obtained from Ecoinvent v3.10 using Brightway2.5 v1.0.6. Moreover, to perform a prospective life cycle assessment, we employ the premise v2.1.2 framework to model future background data using the IMAGE Integrated Assessment Model, following the shared socioeconomic pathway SSP2 (i.e., ‘middle-of-the-road’) under different representative concentration pathways (RCPs). Overall, our results indicate that the biomass-based alternatives demonstrate the best economic and environmental performance. Furthermore, we find that using prospective LCA data can greatly affect the outcome of the analysis, reinforcing the need to accompany standard LCAs with prospective studies to obtain a more comprehensive picture of the process’s potential.

Keywords: propanol production, prospective life cycle assessments, renewable carbon feedstocks

References

Bauer, F., Tilsted, J.P., Pfister, S., Oberschelp, C., Kulionis, V., 2023. Mapping GHG emissions and prospects for renewable energy in the chemical industry. Curr. Opin. Chem. Eng. 39, 100881. https://doi.org/10.1016/j.coche.2022.100881

Vo, C.H., Mondelli, C., Hamedi, H., Pérez-Ramírez, J., Farooq, S., Karimi, I.A., 2021. Sustainability Assessment of Thermocatalytic Conversion of CO2 to Transportation Fuels, Methanol, and 1-Propanol. ACS Sustain. Chem. Eng. 9, 10591–10600. https://doi.org/10.1021/acssuschemeng.1c02805